Daniel:

Attached is a patch to linux-2.6.13.2-lc5 which contains the ide cf driver ln2410sbc-ide-cf.c (in drivers/ide/arm) and an ide hard disk drive ln2410sbc-ide-hdd.c.  Note this patch also contains my modifications to the following drivers:

SD/MMC - reads more reliably mmc cards and also 2GB SD Cards

ln2410sbc_ts (touchscreen for LP35) - modified to work "better" with nano-X

sc2410_buttons (for LP35)

ln2410sbc_js (“joy stick” on LP35) - may cause problems - needs more checking and how to incorporate into nano-X.  Works with one on my boards, but seems to cause the another to fail to boot.

s3c2410_udc (UBS gadget UBS Device Controller for ln2410sbc)

The above drivers had attributes from which one can modify driver parameters without recompiling the drivers.  See the driver source code.  I haven't documented these yet and I may eliminate some of them in the future.

I included a copy of my latest root filesystem rootfs-ide-bb1.1b.gz, the boot loader binary u-boot-ide-tag.bin which I am using and its source uboot-1.1.1-ln2410sbc-0.5-rag.tar.gz. This modification needed to setup the memory region correctly for the compact flash and ide hard drive.  This also has the board configuration file to set up the TAG's need to use bootm.

I had made comments about these modifications and drivers at one time or another on the LittleChip's forums.

The compact flash is assigned to hda, hda1 etc (major 3, minors 0, 1,…) and the hard drive to hdc, hdc1 etc (major 22, minors 0, 1,..) if the compact flash driver is present.

There is a default configuration file (ln2410sbc_cf_defconfig) for the compact flash (without the hard drive) and one file (ln2410_cf_hdd_defconfig) for both the compact flash and an ide hard drive.

In menuconfig, the options for the compact flash and ide hard drive are in

Device Drivers->ATA/ATAPI/MFM/RLL support -->

Good luck,

DicK Grot